MNB28G-24EGP-4GC-M

Managed Ethernet Switch

Managed 24*1000Base T(X) POE + 4*1000Base FX SFP COMBO port Ethernet Switch

 24 порта 1000Base T(X) POE + 4 комбинированных порта Gigabit SFP

 Порты Ethernet POE полностью соответствуют стандарту IEEE802.3af / IEEE802.3at, обеспечивают максимальную мощность 30 Вт на порт.

 Возможность пересылки всех портов на скорости проводной сети обеспечивает неблокирующую пересылку сообщений.

 Автоматический кроссовер MDI/MDI-X для Plug-and-Play

 Поддержка настройки питания POE, мониторинга

 Поддержка автоматического обучения и автоматического устаревания Mac-адреса.

 Поддержка визуального веб-интерфейса, поддержка управления SNMP.

 Поддержка диапазона рабочих температур при полной нагрузке от -20 до 70 ℃.

 Нет вентилятора, конструкция с низким энергопотреблением.

 Блок питания POE составляет 400 Вт.

Introduction

Управляемый HENGSION MNB28G-24EGP-4G-M обеспечивает 4* гигабитных комбинированных порта SFP и 24 порта 10/100/1000BaseT(X) Ethernet POE. Порты Ethernet POE полностью соответствуют стандартам IEEE802.3af и 802.3at и обеспечивают мощность до 30 Вт на порт. Поддержка разделения VLAN, зеркалирования портов и ограничения скорости порта; Поддержка подавления широковещательного шторма, управления потоками, а также централизованного управления и настройки через WEB и SNMP; Управление питанием POE.

Серия MNB28G-POE эффективно снижает стоимость прокладки кабелей и трудозатраты. Широкий диапазон рабочих температур и конструкция защиты от перенапряжения портов идеально подходят для применения в средах с большими расходами, широко используются в системах управления и наблюдения промышленной автоматизации, таких как кампусы, общественные помещения, железнодорожное движение, управление электроэнергией и т. д.

Specification

Managed Ethernet Switch

| Technology |  |
| --- | --- |
| Standards | IEEE 802.3i,802.3u,802.3x, 802.3ab, 802.3az; IEEE802.1Q,802.1D,802.1w,802.1s,802.3ad and LACP,802.1X,802.1ab IEEE802.3af, IEEE802.3at |
| Protocols | STP/RSTP/MSTP, ERPS,IGMP Snooping, MLD snooping, VLAN, Telnet, HTTP, HTTPS, SNMPv1/v2/v3,LLDP,SNTP, SSH ,FTP, QoS |
| Interface |  |
| Gigabit Ethernet port | 10/100/1000 Base-T(X) auto-adaptive RJ45 |
| Gigabit Fiber port | 1000Base-X SFP port |
| Console port | RS232 in RJ45 connector with console cable |
| Power supply port | 5.08 industrial terminal |
| Switching Features |  |
| Processing Type | Store & forward, wire speed switching |
| Switching bandwidth | 196Gbps |
| Packet forwarding speed | 42Mpps |
| MAC Address | 8K |
| Buffer Memory | 4.1Mbit SRAM |
| Priority Queue | 4 |
| VLAN number | 4K |
| VLAN ID | 1-4096 |
| Multicast groups | 256 |
| Software Features |  |
| VLAN | 802.1Q,Vlan, Port-based VLAN |
| Storm Suppression | Broadcast, multicast, and unknown unicast storm suppression |
| Flow control | IEEE802.3X negotiation, back-pressure |
| Multicast protocol | IGMPv1/2/3 Snooping |
| Port Management | Support port mirroring, port isolation, port trunking |
| QoS (Quality of Service) | 802.1p; Support port default priority tags, at least 4 different priority queues per port |
| Security features | MAC address filtering, dynamic or static MAC address learning, Loop detection and Por+MAC binding, support 802.1X port authentication (Radius, local) |
| Traffic Management | Port traffic monitoring and fault event alarming |
| Management | SNMP v1/v2/v3, WEB |
| Management Access | Support Console, Telnet, ssh |
| System, Management | Support system IP setting, restore factory setting, device restart and upgrade |
| File transfer | Support Log output, configuration file backup and input |
| LED Indicator for |  |
| Power, Ethernet port connection & running status, Fiber port connection & running status, POE status |  |
| POE Features |  |
| POE Ports | 24* 10/100/1000Base-TX POE Ethernet ports |
| Power supply mode | Supply power while Power Device detected |
| Bridging mode | Endspan (V+ corresponds to 1,2; V- corresponds to 3,6 ) |
| AF/AT Mode | Support IEEE802.3af/at confirmed PD, provide up to 30Watts per port. |
| Power |  |
| Input Voltage Range | 400w@100-260VAC,50/60HZ |
| POE Power Consumption | AF:15.4W/AT:30W ( each port) |
| Protection | Overload Current Protection; Redundancy protection |
| Mechanical |  |
| Casing | Metal casing |
| Dimension(L*W*H) | 440*291*44.4mm |
| Installation | I U rack mount |
| Weight | 4.2KG |
| Environmental |  |
| Operating Temperature | -20℃～70℃ |
| Storage Temperature | -40℃～+70℃ |
| Relative Humidity | 5～95%,non-condensing |
| Warranty |  |
| Warranty Period | 3 years |

Connection Topology

Managed Ethernet Switch

Ordering Information

| Available Models | Ordering Details |
| --- | --- |
| MNB28G-24EGP-4G-M | Managed, IEEE 802.3AF/AT; 24*10/100/1000M TX POE ports+4*1000Base FX SFP,400W power supply. -20℃～70℃. |

Package Checklist

•  Ethernet switch

•  Power wire

•  Hardware installation guide(printed)

•  Warranty Card

Dimension